SUMMARY: This document details the bill of materials for the INDUSRY 4.0 Pilot project_bel

CONFIDENTIAL

| Part name/Part Number | Specification | Make | Quantity | Function Criticality |
| --- | --- | --- | --- | --- |
| Energy meter | DPM_C501L | Delta | 14 Nos. | NC |
| PLC with communication cable | DVP-14SS211T | Delta | 1 No. | NC |
| Server CPU | HPE Proliant DL380 Gen10 Server | HP | 1 No. | NC |
| Work station | HP Z2 G9 | HP | 1 No. | NC |
| Siemens OPC UA | OPCUA | Siemens | 1 No. | NC |
| Small enclosure | Sheet metal box | -------- | 14 Nos. | NC |
| Power supply | 24V supply | Shavison | 1 No. | NC |
| UPS 1x7.2Ah Battery provides 20 - 75 min | 360Watts / 600VA. | APC | 1 No. | NC |
| Electrical Cabinet | CRCA enclosure | Adharsha | 1 No. | NC |
| 16A MCB- | 16 A, 415 V | SCHNEIDER | 15 Nos. | NC |
| MCB 1Pole 4A | 4 A, 230V | Siemens | 1 No. | NC |
| Isolator switch 2 pole 4A | 4 A, 230V | SCHNEIDER | 1 No. | NC |
| SCHNEIDER MCB 2pole 2A | 2 A, 230V | SCHNEIDER | 15 Nos. | NC |
| RS 485 cable | 2 core twisted | Belden | 340 mtrs | NC |
| 4 Core | 1.5 sqmm | Finolex | 200 mtrs | NC |
| 1 sq mm Wire | Red and Black | Finolex | 200 mtrs | NC |
| Ferrule Lugs and Cable markers as per cable wire size | ----------- | ----------- | 1 Pack | NC |
| 1.5 sq mm, insulated Ring lugsM10 | ----------- | ----------- | 1 Pack | NC |
| 1.5 sq mm insulated Ring lug M12 | ----------- | ----------- | 1 Pack | NC |
| 1 Sq mm, insulated Ring Lugs M8 | ----------- | ----------- | 1 Pack | NC |
| 1.5 sq mm Ferrule lugs Bootlace | ----------- | ----------- | 1 Pack | NC |
| 0.5 sq mm Ferrule lugs Bootlace | ----------- | ----------- | 1 Pack | NC |
| 0.5 sq mm Alphabets markers | ----------- | ----------- | 1 Pack | NC |
| 0.5 sq mm Number markers | ----------- | ----------- | 1 Pack | NC |
| 1.5 sq mm Number markers | ----------- | ----------- | 1 Pack | NC |
| Din rail 35mm | ----------- | ----------- | 5 mtrs | NC |
| M4 Screws, M4 washers & nuts | ----------- | ----------- | 1 Pack | NC |
| M6 Screws, washers & nuts | ----------- | ----------- | 1 Pack | NC |
| Terminal Blocks 2.5Sqmm | KUT 2.5 | Elmex | 100 nos. | NC |
| End clamp and Partitioner | End plate | Elmex | 100 nos. | NC |
| PVC Conduit pipe Flexible 16mm | PG 16 hose | PVC | 300 mtrs | NC |
| Cable Gland- PG16 | PG16 | ----------- | 90Nos. | NC |
| Cable ties (100 mm) | ----------- | ----------- | 1 Pack | NC |
| Cable ties (200 mm) | ----------- | ----------- | 1 Pack | NC |
| LAPP make 6 core shielded twisted pair 0.5 sq mm signal flexible cable | 0.5 sqmm 6 core shielded cable | LAPP | 35 mtrs | NC |
| Casing and caping | 1.5 & 2 inch | ----------- | 70 mtrs | NC |